Below are atomic notes extracted from Module 2a (Number Systems) and Module 2b (Codes). I’ve enforced strict atomicity and evergreen style.

---

title: "Positional number systems assign value based on place"

date: 2026-04-18

tags: [semester-3, scsr2213, digital-logic]

---

# Positional number systems assign value based on place

A positional number system determines a digit’s value using its position and the base of the system.

## Explanation

In any positional system, each digit is multiplied by a power of the base depending on its position. The position relative to the radix point determines whether the exponent is positive or negative. This structure allows compact representation of very large or very small numbers using a fixed set of symbols.

This matters because all digital systems rely on positional representation, especially binary. Without positional weighting, arithmetic operations and conversions between systems would become impractical. The same principle applies across decimal, binary, octal, and hexadecimal systems.

Understanding this concept is foundational since every conversion, arithmetic operation, and encoding method depends on interpreting numbers correctly through positional weights.

## Key Points

- Value = digit × (base^position)

- Left of radix → positive powers

- Right of radix → negative powers

## Example

Convert \(4839.72_{10}\):

(4 × 10³) + (8 × 10²) + (3 × 10¹) + (9 × 10⁰) + (7 × 10⁻¹) + (2 × 10⁻²)

= 4000 + 800 + 30 + 9 + 0.7 + 0.02

= 4839.72

## See Also

- [[binary-number-representation-uses-base-two|Binary Representation]] — applies same positional concept

- [[number-system-conversion-uses-division-and-multiplication|Number System Conversion]] — relies on positional weights

FILENAME: positional-number-systems-assign-value-based-on-place.md

---

title: "Binary representation uses base two with weighted bits"

date: 2026-04-18

tags: [semester-3, scsr2213, digital-logic]

---

# Binary representation uses base two with weighted bits

Binary numbers represent values using only 0 and 1 with powers of two as positional weights.

## Explanation

Binary is the fundamental language of digital systems because hardware naturally operates with two stable states, typically HIGH and LOW voltage. Each binary digit, or bit, corresponds to a power of 2 depending on its position.

This system is efficient for electronic implementation since circuits can reliably distinguish between two states. Every number, instruction, or data structure inside a computer is ultimately stored and processed in binary form.

Understanding binary is critical because all higher-level representations such as hexadecimal, ASCII, and machine instructions are built on top of it.

## Key Points

- Base = 2

- Each position = power of 2

- Only digits: 0 and 1

## Example

Convert \(110100.011_2\) to decimal:

(1×2⁵) + (1×2⁴) + (0×2³) + (1×2²) + (0×2¹) + (0×2⁰)

+ (0×2⁻¹) + (1×2⁻²) + (1×2⁻³)

= 32 + 16 + 4 + 0.25 + 0.125

= 52.375

## See Also

- [[positional-number-systems-assign-value-based-on-place|Positional System]] — explains weighting logic

- [[binary-to-hexadecimal-conversion-uses-grouping|Binary to Hex Conversion]] — groups binary bits

FILENAME: binary-representation-uses-base-two-with-weighted-bits.md

---

title: "Number system conversion uses division and multiplication"

date: 2026-04-18

tags: [semester-3, scsr2213, digital-logic]

---

# Number system conversion uses division and multiplication

Conversion between number systems uses repeated division for integers and repeated multiplication for fractions.

## Explanation

To convert from decimal to another base, the integer part is processed using successive division by the target base, while the fractional part is processed using successive multiplication. The remainders and integer parts extracted form the new number.

This method works because it systematically decomposes the number into powers of the new base. It ensures accuracy and is widely used in both manual computation and algorithm design.

This concept matters because digital systems frequently convert between binary, octal, and hexadecimal representations for efficiency and readability.

## Key Points

- Integer → divide repeatedly

- Fraction → multiply repeatedly

- Stop when precision is sufficient

## Example

Convert 1447₁₀ to hexadecimal:

1447 ÷ 16 = 90 remainder 7

90 ÷ 16 = 5 remainder 10 (A)

5 ÷ 16 = 0 remainder 5

Answer: 5A7₁₆

## See Also

- [[binary-to-hexadecimal-conversion-uses-grouping|Binary to Hex Conversion]] — shortcut method

- [[positional-number-systems-assign-value-based-on-place|Positional System]] — basis of conversion

FILENAME: number-system-conversion-uses-division-and-multiplication.md

---

title: "Binary to hexadecimal conversion uses grouping of four bits"

date: 2026-04-18

tags: [semester-3, scsr2213, digital-logic]

---

# Binary to hexadecimal conversion uses grouping of four bits

Binary numbers are converted to hexadecimal by grouping bits into sets of four starting from the least significant bit.

## Explanation

Since 16 = 2⁴, each hexadecimal digit corresponds exactly to four binary bits. This allows direct conversion without arithmetic, making it efficient and less error-prone.

Grouping begins from the least significant bit (right side) and proceeds left. If needed, leading zeros are added to complete groups. Each group is then mapped to its hexadecimal equivalent.

This matters because hexadecimal provides a compact and human-readable representation of binary data, especially in programming and digital system design.

## Key Points

- Group size = 4 bits

- Start from LSB

- Pad with leading zeros if needed

## Example

Convert 1001100111₂ to hexadecimal:

Group: 0010 0110 0111

= 2 6 7

Answer: 267₁₆

## See Also

- [[binary-representation-uses-base-two-with-weighted-bits|Binary Representation]] — base system

- [[number-system-conversion-uses-division-and-multiplication|Conversion Methods]] — alternative method

FILENAME: binary-to-hexadecimal-conversion-uses-grouping-of-four-bits.md

---

title: "Binary coded decimal represents digits separately in binary"

date: 2026-04-18

tags: [semester-3, scsr2213, digital-logic]

---

# Binary coded decimal represents digits separately in binary

Binary Coded Decimal encodes each decimal digit individually using four binary bits.

## Explanation

Unlike pure binary, BCD does not represent the entire number as a binary value. Instead, each decimal digit is converted separately into its 4-bit binary equivalent. For example, the decimal number 45 is stored as 0100 0101 rather than its binary equivalent 101101.

This approach simplifies display systems and human-readable outputs because each digit remains distinct. However, it is less efficient in storage compared to pure binary.

BCD is widely used in applications such as digital clocks, calculators, and financial systems where exact decimal representation is important.

## Key Points

- Each digit = 4 bits

- Valid range: 0000 to 1001

- 1010 to 1111 are invalid

## Example

Convert 59₁₀ to BCD:

5 → 0101

9 → 1001

Answer: 0101 1001

## See Also

- [[binary-representation-uses-base-two-with-weighted-bits|Binary Representation]] — contrasts with BCD

- [[ascii-encodes-characters-as-binary-values|ASCII Encoding]] — another encoding system

FILENAME: binary-coded-decimal-represents-digits-separately-in-binary.md

---

title: "Gray code ensures only one bit changes between values"

date: 2026-04-18

tags: [semester-3, scsr2213, digital-logic]

---

# Gray code ensures only one bit changes between values

Gray code is a binary numbering system where consecutive values differ by only one bit.

## Explanation

Unlike standard binary, where multiple bits may change between values, Gray code minimizes errors during transitions. This is especially important in electromechanical systems where slight timing differences can cause incorrect readings.

The conversion process involves keeping the most significant bit the same and generating subsequent bits using XOR-like operations with adjacent bits. This ensures only one bit flips at a time.

This matters in applications like rotary encoders and digital sensors, where reliability during transitions is critical.

## Key Points

- Only one bit changes between consecutive values

- Reduces transition errors

- Used in hardware systems

## Example

Binary → Gray:

Binary: 1001

Gray:

- MSB = 1

- Next: 1⊕0 = 1

- Next: 0⊕0 = 0

- Next: 0⊕1 = 1

Result: 1101

## See Also

- [[binary-representation-uses-base-two-with-weighted-bits|Binary Representation]] — base system

- [[parity-bit-detects-errors-using-even-or-odd-counts|Parity Bit]] — another error-related concept

FILENAME: gray-code-ensures-only-one-bit-changes-between-values.md

---

title: "Parity bit detects errors using even or odd counts"

date: 2026-04-18

tags: [semester-3, scsr2213, digital-logic]

---

# Parity bit detects errors using even or odd counts

A parity bit is added to binary data to ensure the number of 1s is either even or odd for error detection.

## Explanation

Parity checking is a simple method used in digital communication to detect transmission errors. By enforcing a rule such as even parity or odd parity, the receiver can verify if the data has been altered during transmission.

If the number of 1s does not match the expected parity, an error is detected. While it cannot correct errors, it provides a quick and low-cost way to detect them.

This method is widely used in memory systems and communication protocols due to its simplicity and efficiency.

## Key Points

- Even parity → total 1s is even

- Odd parity → total 1s is odd

- Detects single-bit errors

## Example

Data: 1010011 (four 1s)

Even parity → already even → parity bit = 0

Result: 01010011

Odd parity → need odd → parity bit = 1

Result: 11010011

## See Also

- [[gray-code-ensures-only-one-bit-changes-between-values|Gray Code]] — reduces transition errors

- [[ascii-encodes-characters-as-binary-values|ASCII Encoding]] — may include parity

FILENAME: parity-bit-detects-errors-using-even-or-odd-counts.md

---

title: "ASCII encodes characters as binary values"

date: 2026-04-18

tags: [semester-3, scsr2213, digital-logic]

---

# ASCII encodes characters as binary values

ASCII represents characters using standardized binary codes, typically 7 bits per character.

## Explanation

ASCII (American Standard Code for Information Interchange) assigns a unique binary number to each character, including letters, digits, and symbols. This allows computers to store and transmit text data consistently.

Each character is mapped to a numeric value, which is then represented in binary or hexadecimal form. This standardization enables interoperability between systems.

ASCII is foundational in computing because it bridges human-readable text and machine-readable binary data.

## Key Points

- Standard: 7-bit encoding

- Supports letters, digits, symbols

- Often extended to 8 bits with parity

## Example

Character: 'm'

ASCII (decimal): 109

Binary: 1101101

Hex: 6D

## See Also

- [[binary-coded-decimal-represents-digits-separately-in-binary|BCD]] — digit encoding alternative

- [[parity-bit-detects-errors-using-even-or-odd-counts|Parity Bit]] — used with ASCII

FILENAME: ascii-encodes-characters-as-binary-values.md

Sources:

- Module 2a Number Systems

- Module 2b Data Organization Codes

If you want, I can continue with Module 2c (signed numbers, complements, arithmetic) next, same format.

Alright, continuing cleanly into Module 2c (Arithmetic + Signed Numbers). Same strict atomic structure, no fluff.

---

title: "Unsigned binary integers represent only non negative values"

date: 2026-04-18

tags: [semester-3, scsr2213, digital-logic]

---

# Unsigned binary integers represent only non negative values

Unsigned binary numbers represent only zero and positive values because all bits are used for magnitude.

## Explanation

In an unsigned representation, every bit contributes to the value of the number, meaning there is no dedicated sign indicator. This allows the full range of values to be used for magnitude, maximizing the representable positive range.

The range of an n-bit unsigned number is from 0 to \(2^n - 1\). This makes it ideal for quantities that cannot be negative, such as memory addresses, counts, or sizes.

This matters because using unsigned representation incorrectly for signed data will produce completely wrong interpretations, especially in low-level programming and hardware systems.

## Key Points

- No sign bit

- Range: 0 to \(2^n - 1\)

- All bits used for magnitude

## Example

For 8-bit unsigned:

Max value = \(2^8 - 1 = 255\)

Binary: 11111111 = 255

Binary: 00000000 = 0

## See Also

- [[sign-magnitude-representation-separates-sign-and-value|Sign Magnitude]] — introduces sign handling

- [[twos-complement-simplifies-negative-number-representation|Twos Complement]] — standard signed method

FILENAME: unsigned-binary-integers-represent-only-non-negative-values.md

---

title: "Sign magnitude representation separates sign and value"

date: 2026-04-18

tags: [semester-3, scsr2213, digital-logic]

---

# Sign magnitude representation separates sign and value

Sign-magnitude representation uses the most significant bit as a sign and the remaining bits as magnitude.

## Explanation

In this system, the leftmost bit (MSB) indicates the sign: 0 for positive and 1 for negative. The remaining bits represent the absolute value of the number.

While this method is intuitive, it introduces complications such as having two representations for zero (+0 and -0). It also makes arithmetic operations more complex because the sign and magnitude must be handled separately.

This matters because although it is simple conceptually, it is rarely used in modern systems due to inefficiency and ambiguity.

## Key Points

- MSB = sign bit

- Remaining bits = magnitude

- Two zeros exist (+0 and -0)

## Example

Represent +25 and -25 in 8-bit:

+25 → 00011001

-25 → 10011001

## See Also

- [[ones-complement-inverts-bits-to-represent-negative-values|Ones Complement]] — improves sign handling

- [[twos-complement-simplifies-negative-number-representation|Twos Complement]] — standard system

FILENAME: sign-magnitude-representation-separates-sign-and-value.md

---

title: "Ones complement inverts bits to represent negative values"

date: 2026-04-18

tags: [semester-3, scsr2213, digital-logic]

---

# Ones complement inverts bits to represent negative values

One’s complement represents negative numbers by flipping all bits of the positive value.

## Explanation

In this system, positive numbers are represented normally, while negative numbers are obtained by inverting every bit (0 becomes 1, and 1 becomes 0). This creates a mirror representation of the number.

However, like sign-magnitude, it still has two representations for zero: all zeros (+0) and all ones (-0). This redundancy complicates arithmetic operations and system design.

This method is historically important but largely replaced by two’s complement due to its limitations.

## Key Points

- Negative = bitwise inversion

- Two zeros exist

- Used before two’s complement

## Example

+7 → 00000111

-7 (1’s complement) → 11111000

## See Also

- [[sign-magnitude-representation-separates-sign-and-value|Sign Magnitude]] — earlier method

- [[twos-complement-simplifies-negative-number-representation|Twos Complement]] — improved system

FILENAME: ones-complement-inverts-bits-to-represent-negative-values.md

---

title: "Twos complement simplifies negative number representation"

date: 2026-04-18

tags: [semester-3, scsr2213, digital-logic]

---

# Twos complement simplifies negative number representation

Two’s complement represents negative numbers by inverting all bits and adding one.

## Explanation

Two’s complement solves the major issues found in sign-magnitude and one’s complement systems. By adding one after inversion, it eliminates the problem of having two zeros and enables simpler arithmetic operations.

This system allows addition and subtraction to be performed using the same binary addition circuitry, making it highly efficient for hardware implementation.

This is the standard representation used in modern computers because it simplifies design and ensures consistent behavior.

## Key Points

- Negative = invert bits + 1

- Only one zero exists

- Simplifies arithmetic operations

## Example

Convert +25 to -25 (8-bit):

+25 → 00011001

1’s complement → 11100110

+1 → 11100111

So, -25 = 11100111

## See Also

- [[ones-complement-inverts-bits-to-represent-negative-values|Ones Complement]] — previous method

- [[binary-addition-ignores-final-carry-in-twos-complement|Binary Addition]] — uses this system

FILENAME: twos-complement-simplifies-negative-number-representation.md

---

title: "Binary addition ignores final carry in twos complement"

date: 2026-04-18

tags: [semester-3, scsr2213, digital-logic]

---

# Binary addition ignores final carry in twos complement

In two’s complement arithmetic, the final carry out of the most significant bit is discarded.

## Explanation

When performing binary addition in two’s complement, any carry that goes beyond the fixed bit width is ignored. This ensures the result stays within the defined number of bits.

This rule allows subtraction to be performed as addition of a negative number, simplifying circuit design. It also ensures consistent results without requiring separate subtraction logic.

This matters because all modern processors rely on this rule for efficient arithmetic operations.

## Key Points

- Ignore final carry

- Same circuit for addition and subtraction

- Works with fixed bit width

## Example

(+60) + (-30):

+60 → 00111100

-30 → 11100010

Sum:

00111100

+11100010

= 1 00011110

Ignore carry → 00011110 = 30

## See Also

- [[twos-complement-simplifies-negative-number-representation|Twos Complement]] — defines negative numbers

- [[binary-subtraction-uses-twos-complement-addition|Binary Subtraction]] — uses same process

FILENAME: binary-addition-ignores-final-carry-in-twos-complement.md

---

title: "Binary subtraction uses twos complement addition"

date: 2026-04-18

tags: [semester-3, scsr2213, digital-logic]

---

# Binary subtraction uses twos complement addition

Binary subtraction is performed by adding the two’s complement of the subtrahend.

## Explanation

Instead of subtracting directly, digital systems convert the number being subtracted into its two’s complement form and then perform addition. This eliminates the need for separate subtraction hardware.

This method works because two’s complement encodes negative numbers in a way that integrates seamlessly with binary addition. It simplifies processor design and improves efficiency.

This approach is fundamental in computer arithmetic and is used in virtually all modern computing systems.

## Key Points

- Subtraction = addition of negative

- Use two’s complement

- No separate subtraction logic needed

## Example

Compute 17 - 9:

+17 → 00010001

+9 → 00001001

Two’s complement of 9:

11110111

Add:

00010001

+11110111

= 1 00001000

Ignore carry → 00001000 = 8

## See Also

- [[twos-complement-simplifies-negative-number-representation|Twos Complement]] — enables method

- [[binary-addition-ignores-final-carry-in-twos-complement|Binary Addition]] — operation used

FILENAME: binary-subtraction-uses-twos-complement-addition.md

Source:
Module 2c Arithmetic Operations

---

title: "Signed number ranges depend on representation method"

date: 2026-04-18

tags: [semester-3, scsr2213, digital-logic]

---

# Signed number ranges depend on representation method

The range of representable values in binary depends on how signed numbers are encoded.

## Explanation

Different signed number systems allocate bits differently between sign and magnitude, which directly affects the range of values that can be represented. In sign-magnitude and one’s complement systems, one bit is reserved for the sign, leaving the rest for magnitude.

Because of this, both systems produce symmetric ranges around zero, but they suffer from duplicate representations of zero. This reduces efficiency and complicates arithmetic operations.

Understanding range differences is critical because incorrect assumptions about range lead to overflow errors and incorrect interpretations of binary values in computations.

## Key Points

- Sign-magnitude and 1’s complement:

Range: \(-(2^{n-1}-1)\) to \(+(2^{n-1}-1)\)

- Two representations of zero exist

## Example

For 4-bit system:

Range = -(2³ - 1) to +(2³ - 1)

= -7 to +7

Binary examples:

+7 → 0111

-7 → 1111

## See Also

- [[twos-complement-simplifies-negative-number-representation|Twos Complement]] — different range behavior

- [[unsigned-binary-integers-represent-only-non-negative-values|Unsigned Numbers]] — no sign involved

FILENAME: signed-number-ranges-depend-on-representation-method.md

---

title: "Twos complement range is asymmetric around zero"

date: 2026-04-18

tags: [semester-3, scsr2213, digital-logic]

---

# Twos complement range is asymmetric around zero

Two’s complement has an asymmetric range because it represents one extra negative value.

## Explanation

In two’s complement, the most significant bit is treated as a weighted negative value, which shifts the range. Instead of having equal positive and negative limits, the system allows one additional negative number.

This eliminates the duplicate zero problem found in other systems and simplifies arithmetic operations. However, it introduces asymmetry that must be considered when interpreting results.

This matters because overflow detection and range checking depend heavily on understanding this asymmetry.

## Key Points

- Range: \(-2^{n-1}\) to \(2^{n-1}-1\)

- Only one zero

- One extra negative value

## Example

For 4-bit system:

Range = -8 to +7

Binary:

-8 → 1000

+7 → 0111

## See Also

- [[signed-number-ranges-depend-on-representation-method|Signed Range]] — comparison with other systems

- [[binary-addition-ignores-final-carry-in-twos-complement|Binary Addition]] — operates within this range

FILENAME: twos-complement-range-is-asymmetric-around-zero.md

---

title: "Overflow occurs when binary results exceed representable range"

date: 2026-04-18

tags: [semester-3, scsr2213, digital-logic]

---

# Overflow occurs when binary results exceed representable range

Overflow happens when the result of a binary operation exceeds the allowed range of the system.

## Explanation

In signed binary arithmetic, overflow occurs when the computed result cannot be represented within the fixed number of bits. This typically happens when adding two numbers of the same sign produces a result with a different sign.

Unlike carry, which is ignored in two’s complement arithmetic, overflow indicates a logical error in the result. It must be detected and handled properly in systems to avoid incorrect computations.

This is critical because processors rely on overflow detection flags to ensure correctness in arithmetic operations.

## Key Points

- Occurs in signed arithmetic

- Same sign inputs → different sign output = overflow

- Carry does NOT indicate overflow

## Example

4-bit system:

+7 → 0111

+4 → 0100

Sum:

0111

+0100

= 1011

Result = -5 (incorrect) → overflow occurred

## See Also

- [[binary-addition-ignores-final-carry-in-twos-complement|Binary Addition]] — where overflow can occur

- [[twos-complement-range-is-asymmetric-around-zero|Twos Complement Range]] — limits that cause overflow

FILENAME: overflow-occurs-when-binary-results-exceed-representable-range.md

---

title: "Same binary pattern can represent different values depending on interpretation"

date: 2026-04-18

tags: [semester-3, scsr2213, digital-logic]

---

# Same binary pattern can represent different values depending on interpretation

A binary pattern has different meanings depending on whether it is interpreted as unsigned or signed.

## Explanation

Binary data itself has no inherent meaning; interpretation depends on the encoding scheme used. The same bit pattern can represent a positive number in unsigned form or a negative number in two’s complement form.

This distinction is essential in computing systems because incorrect interpretation leads to logical errors. For example, memory data or register values must always be interpreted in the correct context.

Understanding this concept is crucial when debugging low-level programs or working with hardware systems.

## Key Points

- Binary is just a pattern

- Meaning depends on interpretation

- Signed vs unsigned changes value

## Example

8-bit binary: 11111111

Unsigned → 255

Two’s complement → -1

## See Also

- [[unsigned-binary-integers-represent-only-non-negative-values|Unsigned Numbers]] — one interpretation

- [[twos-complement-simplifies-negative-number-representation|Twos Complement]] — alternative interpretation

FILENAME: same-binary-pattern-can-represent-different-values-depending-on-interpretation.md